CNT 4703C

Date:  June 24th, 2016	 	 	 	 	 	Name: Brian Batinchok

Section:  CMB-16 (Summer 2016)	 	 	 	 	ID: BR090814/B2232953

# Exam 1

Instructions:  There are Multiple-Choice, Fill-in-the-Blank, and Free-Response questions on 5 pages.

PART 1--Multiple-Choice Questions: circle the letter which corresponds to the correct answer from those listed.  When a question is indicated as having more than one correct answer, circle only that many letters.

- The process of a web server adding a TCP header to the contents of a web page, followed by adding an IP header and then adding a data link header and trailer is an example of what?

    - Data encapsulation 		(encapsulation is the process of adding headers)

    - Same-layer interaction

    - OSI model

    - All of these answers are correct.

- Which OSI layer defines the functions of logical network-wide addressing and routing?

    - Layer 1

    - Layer 2

    - Layer 3 		(network layer)

    - Layer 4

    - Layer 5, 6, or 7

- Which OSI layer defines the standards for cabling and connectors?

    - Layer 1 		(physical layer)

    - Layer 2

    - Layer 3

    - Layer 4

    - Layer 5, 6, or 7

- Data can flow only in both directions all of the times in a _________ mode.

    - Simplex

    - half-duplex

    - full-duplex 	(full duplex allows sending a receiving simultaneously)

    - None of the choices are correct

- In a _____________ communication, the media is dedicated.

    - point-to-point 	(direct link between the two points, unlike broadcasting)

    - multipoint

    - mesh

    - None of the choices are correct

- In a fully connected mesh topology with six computers, we need ________ links.

a. 	6

    - 36

    - 15 		formula: n(n-1)/2

    - None of the choices are correct

- Host A is a PC, connected to switch SW1 and assigned to VLAN 1. Which of the following are typically assigned an IP address in the same subnet as host A? (Select two answers)

    - The local router’s WAN interface

    - The local router’s LAN interface

    - All other hosts attached to the same switch

    - Other hosts attached to the same switch and also in VLAN 1

- Why does the formula for the number of hosts per subnet (2H – 2) require the subtraction of two hosts?

    - To reserve two addresses for redundant default gateways (routers)

    - To reserve the two addresses required for DHCP operation

    - To reserve addresses for the subnet ID and default gateway (router)

    - To reserve addresses for the subnet broadcast address and subnet ID

- A Class B network needs to be subnetted such that it supports 100 subnets and 100 hosts/subnet. Which of the following answers list a workable combination for the number of network, subnet, and host bits? (Select two answers.)

    - Network = 16, subnet = 7, host = 7

    - Network = 16, subnet = 8, host = 8

    - Network = 16, subnet = 9, host = 7

    - Network = 8, subnet = 7, host = 17

- Which of the following are private IP networks? (Select two answers.)

    - 172.31.0.0 			Private IP ranges:

    - 172.32.0.0 			24-bit: 10.0.0.0 – 10.255.255.255

    - 192.168.255.0 			20-bit: 172.16.0.0 – 172.31.255.255

    - 192.1.168.0 			16-bit: 192.168.0.0 – 192.168.255.255

    - 11.0.0.0

- Which of the following are not valid Class B network IDs?

    - 130.0.0.0 			Class B properties:

    - 191.255.0.0 			128-191 first octet

    - 128.0.0.0 			0-255 second octet

    - 150.255.0.0 			0’s in final two octets

    - All are valid Class B network IDs

- Which of the following are true about IP address 172.16.99.45’s IP network? (Select two answers.) a. 	The network ID is 172.0.0.0.

    - The network is a Class B network.

    - The default mask for the network is 255.255.255.0.

    - The number of host bits in the unsubnetted network is 16.

- Which of the following is a network broadcast address?

    - 10.1.255.255

    - 192.168.255.1

    - 224.1.1.255

    - 172.30.255.255 			ends is 255

- Which of the following answers lists the prefix (CIDR) format equivalent of 255.255.255.240?

a. 	/26

    - /28 				255.255.255.240 -> 8+8+8+4 (binary) = 28

    - /27

    - /30

    - /29

- Working at the help desk, you receive a call and learn a user’s PC IP address and mask (10.55.66.77, mask 255.255.255.0). When thinking about this using classful logic, you determine the number of network (N), subnet (S), and host (H) bits. Which of the following is true in this case?

a. 	N=12

    - S=12

    - H=8

    - S=8

    - N=24

- An engineer is thinking about the following IP address and mask using classless IP addressing logic:

10.55.66.77, 255.255.255.0. Which of the following statements are true when using classless addressing logic? (Choose two.)

    - The network part’s size is 8 bits.

    - The prefix length is 24 bits. 		Prefix length = binary 1’s of mask -> 8+8+8 = 24

    - The prefix length is 16 bits.

    - The host part’s size is 8 bits. 		32 bits – 24 bits = 8 bits

- Which of the following statements is true about classless IP addressing concepts?

    - Uses a 128-bit IP address

    - Applies only for Class A and B networks

    - Separates IP addresses into network, subnet, and host parts

    - Ignores Class A, B, and C network rules

- Which of the following masks, when used as the only mask within a Class B network, would supply enough subnet bits to support 100 subnets? (Choose two.)

    - /24 				mask = binary 1’s of subnet -> 8+8+8 = 24

    - 255.255.255.252

    - /20

    - 255.255.252.0

- To set timeout for your console to 5 minutes and 10 seconds, the command “lin con 0” is followed by:

    - Exec-timeout 310

    - Exec-timeout 5 10 		exec-timeout (minutes) (seconds)

    - Exec-timeout 10 5

    - None of the choices are correct

- The command to display the current configuration of a router (or switch) is _”show running-config”_

System:running-config is replacing show running-config but show-running config will continue to be supported.

Part 2--Free-Response Questions:

- Find the Network ID and Network Broadcast of the following:

|  | IP Address | Class | Network Octets | Host Octets | Network ID | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.1.1.1 | A | 1 | 3 | 1.0.0.0 | 1.255.255.255 |
| 2 | 128.1.6.5 | B | 2 | 2 | 128.1.0.0 | 128.1.255.255 |
| 3 | 200.1.2.3 | C | 3 | 1 | 200.1.2.0 | 200.1.2.255 |
| 4 | 192.192.1.1 | C | 3 | 1 | 192.192.1.0 | 192.192.2.255 |
| 5 | 126.5.4.3 | A | 1 | 3 | 126.0.0.0 | 126.255.255.255 |
| 6 | 200.1.9.8 | C | 3 | 1 | 200.1.9.0 | 200.1.9.255 |
| 7 | 192.0.0.1 | C | 3 | 1 | 192.0.0.0 | 192.0.0.255 |
| 8 | 191.255.1.47 | B | 2 | 2 | 191.255.0.0 | 191.255.255.255 |
| 9 | 223.223.0.1 | C | 3 | 1 | 223.223.0.0 | 223.223.0.255 |

- Assign IP addresses to the interfaces of the routers in the network using private class C network.

The IP address and the subnet mask of:

Interface F1/0 of R1: 192.168.0.0/27

Interface F1/1 of R1:  192.168.0.32/27

Interface F1/0 of R2:  192.168.0.128/27

Interface F3/0 of R2:  192.168.0.160/27

What are the subnet IDs and their broadcast addresses?

Interface F1/0 of R1:

- Subnet ID: 192.168.0.0

- Broadcast Address: 192.168.0.31

Interface F1/1 of R1:

- Subnet ID: 192.168.0.32

- Broadcast Address: 192.168.0.63

Interface F1/0 of R2:

- Subnet ID:  192.168.0.128

- Broadcast Address: 192.168.0.159

Interface F3/0 of R2:

- Subnet ID: 192.168.0.160

- Broadcast Address: 192.168.0.191

Corrections:

The IP address and the subnet mask of  Interface F1/0 of R1: 192.168.1.1 255.255.255.0  Interface F1/1 of R1: 192.168.2.1 255.255.255.0  Interface F1/0 of R2: 192.168.3.1 255.255.255.0  Interface F3/0 of R2: 192.168.1.2 255.255.255.0 What are the subnet IDs and their broadcast addresses? Subnet of F1/0 of R1 and F3/0 of R2: 192.168.1.0, 192.168.1.255 Subnet of F1/1 of R1: 192.168.2.0, 192.168.2.255 Subnet of F1/0 of R2: 192.168.3.0, 192.168.3.255 + 4th row of the table